TECHNICAL CHANGE REPORT

DragonBox Pyra
Kernel 5.6.19

Cortex-M4 / IPU integration, AMMU safety controls,
PRM reset corrections and platform stability fixes

| BASELINE | 5.6.19-pyradef · Debian packaging revision -10 |
| --- | --- |
| MODIFIED BUILD | 5.6.19-pyradef-m4test-m4clean · Pyra M4 v6 |
| SCOPE | 15 changed source paths · 14 modified · 1 added |
| PLATFORM | DragonBox Pyra V5.3 · TI OMAP5432 / ARMv7 |
| SOURCE BASIS | Integrated source patch and verified changed-file manifest |
| ISSUED | 26 October 2026 |

DOCUMENT PURPOSE

A structured, file-by-file account of all source changes introduced by the experimental M4 kernel project. Installed boot artifacts and explicit non-changes are documented separately so the scope remains auditable.

VERIFICATION BOUNDARY The source inventory is grounded in the integrated project patch and manifest. A successful source comparison or kernel build does not, on its own, establish successful M4 firmware execution or validate all Pyra hardware functions.

# 01  Executive overview

The modified kernel introduces a guarded remoteproc path for the OMAP5432 IPU Cortex-M4 subsystem while preserving the original Pyra Linux environment and its PowerVR graphics stack.

## Change areas

•  M4 enablement: Device Tree-aware remoteproc driver, separate M4-enabled DTB, mailbox and remoteproc configuration.

•  CPU0-only safety: Explicit reset sequencing, AMMU Small Page 0 mapping, readback, rollback and one-start-per-boot guard.

•  PRM / Letux: Selected Letux 7.3 reset-deassert sequencing fixes and opt-in IPU reset diagnostics/quirk.

•  Platform fixes: MMC timeout overflow, charger property parsing, TILER allocation errors and DWC3 USB/OTG handling.

•  Build and isolation: ARM compiler-flag ordering, SELinux export header compatibility, independent kernel release and modules.

## Source change matrix

| # | Kernel source path | Change |
| --- | --- | --- |
| 01 | arch/arm/Makefile | ARM kernel build flags |
| 02 | arch/arm/boot/dts/Makefile | Additional DTB target |
| 03 | arch/arm/boot/dts/omap5-letux-cortex15-v5.3+pyra-v5.3-m4test.dts | New M4-enabled Pyra DTS |
| 04 | arch/arm/boot/dts/omap5.dtsi | OMAP5 IPU node |
| 05 | arch/arm/configs/pyra_defconfig | Remoteproc/mailbox config |
| 06 | drivers/gpu/drm/omapdrm/omap_dmm_tiler.c | ENOMEM error propagation |
| 07 | drivers/iommu/omap-iommu.c | Header dependencies |
| 08 | drivers/mmc/host/omap_hsmmc.c | Timeout arithmetic |
| 09 | drivers/power/supply/bq2429x_charger.c | DT property conditionals |
| 10 | drivers/remoteproc/Kconfig | OMAP5 help text |
| 11 | drivers/remoteproc/omap_remoteproc.c | Remoteproc, CPU0, AMMU |
| 12 | drivers/remoteproc/omap_remoteproc.h | Mailbox message IDs |
| 13 | drivers/soc/ti/omap_prm.c | PRM reset sequence |
| 14 | drivers/usb/dwc3/dwc3-omap.c | USB/OTG regulator state |
| 15 | security/selinux/include/classmap.h | Exported-header guard |

# 02  Device Tree and kernel configuration

These changes introduce an opt-in IPU description and build target without modifying the original Pyra board DTS.

## 02  arch/arm/boot/dts/Makefile

•  Adds omap5-letux-cortex15-v5.3+pyra-v5.3-m4test.dtb to the CONFIG_SOC_OMAP5 Device Tree build targets.

•  Retains the original Pyra V5.3 DTB target.

## 03  arch/arm/boot/dts/omap5-letux-cortex15-v5.3+pyra-v5.3-m4test.dts

•  NEW source file (10 lines).

•  Includes the original V5.3 board DTS.

•  Overrides only the IPU node with: &ipu { status = "okay"; };

•  The original board DTS remains unmodified.

## 04  arch/arm/boot/dts/omap5.dtsi

•  Adds an IPU node at physical address 0x55020000, initially disabled.

•  Compatible string: "ti,omap5-ipu".

•  Internal L2 SRAM region: 0x55020000, length 0x10000 (64 KiB); register name "l2ram".

•  Connects the IPU IOMMU (&mmu_ipu), two reset controls (&prm_core 0 and &prm_core 1), IPU clock and OMAP mailbox.

•  Firmware name: "omap5-ipu-fw.xem4".

•  No reserved-memory node or memory-region reference is added.

## 05  arch/arm/configs/pyra_defconfig

•  Changes CONFIG_LOCALVERSION from "-pyradef" to "-pyradef-m4test".

•  Enables CONFIG_OMAP2PLUS_MBOX=y.

•  Enables CONFIG_REMOTEPROC=y.

•  Enables CONFIG_OMAP_REMOTEPROC=y.

•  Other defconfig lines are unchanged by the combined source patch.

•  The actual distributed build additionally supplies LOCALVERSION=-m4clean, producing the release string 5.6.19-pyradef-m4test-m4clean.

# 03  Remoteproc and M4 execution controls

The OMAP5 IPU is represented as one Linux remoteproc device. The experimental start path intentionally targets CPU0 only; CPU1 remains held in reset.

## 10  drivers/remoteproc/Kconfig

•  Updates the OMAP remoteproc help text to mention OMAP4 and OMAP5 Device Tree-based IPU support instead of claiming OMAP4-only support.

•  The actual feature selection is made in pyra_defconfig.

## 11  drivers/remoteproc/omap_remoteproc.c

•  Replaces the earlier platform-data-oriented OMAP driver with a Device Tree-aware driver based on the Linux 5.7 implementation, adapted to Linux 5.6 interfaces.

•  Uses Device Tree compatible matches, firmware-name, reset controls, clocks, mailbox channel and optional timer resources.

•  Maps internal remote-processor memories and implements the remoteproc device-address-to-host-address translation callback.

•  Supports firmware loading, start/stop, mailbox signaling, watchdog/timer integration, runtime PM and suspend/resume paths where applicable.

•  Sets rproc->auto_boot = false; the M4 firmware is not started automatically when remoteproc registers.

•  Adapts timer set_load() to the three-argument kernel API, da_to_va() to an int length argument, and boot-address logging to a 32-bit 0x%08x format.

•  Adds the built-in module parameter omap_remoteproc.pyra_cpu0_only=1 for the OMAP5 IPU. The parameter defaults to disabled.

•  When CPU0-only mode is requested, obtains two independent reset controls, asserts both, and verifies both are held at probe.

•  On a firmware start, reasserts CPU0 and CPU1 reset, performs AMMU preflight, releases CPU0 only, and verifies the expected post-start reset state (CPU0 released, CPU1 held).

•  Refuses an OMAP5 IPU firmware start if CPU0-only mode is not active; this is an experimental safety constraint, not generic dual-core IPU support.

•  On stop, asserts CPU0 and ensures CPU1 remains held.

•  Adds an atomic one-attempt-per-kernel-boot guard. A second firmware-start attempt is rejected; reboot is required.

•  Implements a guarded AMMU Small Page 0 preparation:

–  Requires CPU0-only mode and both CPU reset lines asserted.

–  Checks RM_IPU_RSTCTRL at 0x4AE06910 equals 0x00000003 before allowing the transition.

–  Reads IPU configuration registers through the host-visible 0x55080000 region, including Page 0 and Page 1 address, translation and policy registers.

–  Accepts only known Page 0 values (ADDR=0, XLTE=0 or 0x55020000, POLICY=0x0b) and known Page 1 values (ADDR=0x40000000, XLTE=0x55080000, POLICY=0x09), together with expected MMU/cache-maintenance state.

–  If necessary, writes 0x55020000 to the Page 0 XLTE register at host physical address 0x550809A0, mapping the M4-local address 0 to the internal IPU L2 SRAM.

–  Reads back the written value and rechecks relevant registers and CPU reset states before CPU0 release.

–  On failed verification, attempts to restore the original XLTE value, keeps CPU0 in reset, and reports an error.

–  Does not write external Linux IOMMU page tables, DDR, CPU1 reset from the AMMU helper, or cache maintenance registers.

•  Retains the Letux PRM changes implemented separately in drivers/soc/ti/omap_prm.c; the AMMU helper does not replace the PRM reset driver.

•  Removes historical experimental diagnostic branches and their boot-argument switches from the final clean driver.

## 12  drivers/remoteproc/omap_remoteproc.h

•  Updates the source header/SPDX metadata and copyright years.

•  Adds mailbox message identifiers for automatic/system suspend, suspend acknowledgement/cancellation, and end-of-message marker: RP_MBOX_SUSPEND_AUTO   = 0xFFFFFF10 RP_MBOX_SUSPEND_SYSTEM = 0xFFFFFF11 RP_MBOX_SUSPEND_ACK    = 0xFFFFFF12 RP_MBOX_SUSPEND_CANCEL = 0xFFFFFF13 RP_MBOX_END_MSG       = 0xFFFFFF14

CRITICAL AMMU MAPPING The controlled mapping is M4 local address 0x00000000 → physical IPU L2 SRAM 0x55020000, using the Small Page 0 XLTE register at 0x550809A0. The code checks reset states and known register patterns before CPU0 release.

# 04  Reset management and platform stability

## 13  drivers/soc/ti/omap_prm.c

•  Adds the module parameter omap_prm.pyra_ipu_prm_trace, disabled by default, for read-only IPU CORE PRM snapshots.

•  Adds the module parameter omap_prm.pyra_ipu_prm_assert_quirk, disabled by default, for an explicitly requested IPU CORE reset assertion during PRM probe.

•  Restricts the optional quirk to the IPU CORE PRM instance (base 0x4AE06700) and rst_map_012.

•  Preserves unrelated RSTCTRL bits via read-modify-write, followed by a readback to flush the posted write.

•  In reset deassertion, returns early if the requested reset is already released.

•  After clearing a reset bit, uses readl_relaxed_poll_timeout_atomic() to wait for RSTCTRL to reflect the cleared state.

•  If a reset status register exists, additionally waits for the corresponding RSTST bit using atomic polling.

•  These are retained selected Letux 7.3 reset-sequencing corrections; they are NOT a wholesale import of Letux 7.3.

•  Optional tracing logs PWRSTCTRL, PWRSTST, RSTCTRL and RSTST before/after reset deassertion and at relevant probe points.

## 06  drivers/gpu/drm/omapdrm/omap_dmm_tiler.c

•  Sets ret = -ENOMEM if the TILER refill DMA allocation fails, before entering the existing error cleanup path.

•  Fixes the returned error code; it does not change the PowerVR rendering pipeline.

## 07  drivers/iommu/omap-iommu.c

•  Adds three header includes: linux/atomic.h, linux/moduleparam.h, and linux/string.h.

•  No executable IOMMU function or mapping logic is changed by the final combined patch in this file.

•  Earlier diagnostic IOMMU experiments are NOT part of this clean source snapshot.

## 08  drivers/mmc/host/omap_hsmmc.c

•  Casts busy_timeout to u64 before multiplying by NSEC_PER_MSEC.

•  Prevents 32-bit intermediate arithmetic overflow in the busy response timeout calculation.

## 09  drivers/power/supply/bq2429x_charger.c

•  Removes accidental trailing semicolons from two of_property_read_u32() conditionals.

•  Applies precharge-current and charge-termination-current values only when their optional Device Tree properties were read successfully, rather than potentially using an uninitialized local value.

## 14  drivers/usb/dwc3/dwc3-omap.c

•  Checks whether the VBUS regulator is enabled before disabling it when the USB ID signal floats.

•  Initializes both USB and USB_HOST extcon states, including explicit OFF/FLOAT states when no cable is present.

•  Uses devm_regulator_get_optional() for VBUS: tolerates -ENODEV but preserves other errors, including probe deferral.

•  Depopulates child devices on an IRQ setup failure before unwinding runtime PM.

# 05  Build and exported-header compatibility

## 01  arch/arm/Makefile

•  Moves -msoft-float into the earlier ARM KBUILD_CFLAGS definition, so compiler feature probes and ordinary kernel C compilation both use the correct no-hardware-FP kernel ABI.

•  Removes the duplicate later -msoft-float occurrence.

•  This is a build-flag ordering fix, NOT an enablement of NEON in the kernel and NOT a change to the Pyra userspace ABI.

## 15  security/selinux/include/classmap.h

•  Restricts the PF_MAX compile-time address-family check to builds where __EXPORTED_HEADERS__ is not defined.

•  This is a header/export build-compatibility change.

# 06  Built artifacts and boot integration

The following differences are produced by the build and installation workflow. They are not additional source-file modifications.

| RELEASE | Original: 5.6.19-pyradef ¦ New: 5.6.19-pyradef-m4test-m4clean |
| --- | --- |
| MODULES | /lib/modules/5.6.19-pyradef-m4test-m4clean/ (946 .ko files in supplied staging archive) |
| KERNEL IMAGE | /boot/vmlinuz-5.6.19-pyradef-m4test-m4clean |
| M4 DEVICE TREE | Additional M4-enabled DTB, installed as ...-m4clean.dtb; generated from the -m4test.dts source |
| INITRAMFS | Target-generated /boot/initrd.img-5.6.19-pyradef-m4test-m4clean, where the boot entry requires it |
| BOOT ENTRY | LABEL pyra-m4clean in /boot/extlinux/user.cfg |
| BOOT ARGUMENT | omap_remoteproc.pyra_cpu0_only=1 |

## PowerVR compatibility

The combined patch does not modify the original PowerVR SGX driver source or native EGL/GLES2 userspace libraries. The staged module tree includes the DDK 1.14.3699939 pvrsrvkm_omap5_sgx544_116.ko module built for the new kernel release. Hardware compatibility still requires on-device testing.

## Recovery and default boot

The original extlinux entries l0, l0r and l0h remain available. In the user-shared configuration, extlinux.conf contained both “default pyra-m4clean” and “default l0”, so the active kernel remained 5.6.19-pyradef. That configuration conflict is not a source change in the modified kernel.

## Separate M4 firmware

The test firmware-v12 project is a separate Cortex-M4 executable, not a kernel source modification and not part of the kernel installer. The Device Tree firmware-name is omap5-ipu-fw.xem4.

# 07  Explicit exclusions and verification limits

•  The original Pyra V5.3 board DTS is unchanged; a separate M4-enabled DTS is added.

•  No modifications to the PowerVR SGX driver source or the native userspace graphics stack.

•  No Mesa, GL4ES or GLX replacement is introduced.

•  No general-purpose DDR AMMU mapping, reserved-memory/CMA allocation or RPMsg application is introduced.

•  No routine CPU1 startup is enabled in CPU0-only mode.

•  No Phase 9.13+ mailbox IPC feature is included in this v6 snapshot.

•  Only selected PRM reset fixes are carried over; this is not a complete Letux 7.3 kernel port.

•  Source consistency does not establish successful M4 firmware execution or stable PowerVR, USB, MMC/F2FS, suspend or charging operation.

•  The earlier reported uname -r value was 5.6.19-pyradef; activation of the new kernel was not independently established in that output.

# 08  Provenance and integrity record

The authoritative comparison baseline is the original Pyra kernel source with the Debian packaging revision -10 diff applied. The integrated source distribution was previously checked against that baseline with zero patch fuzz; the 15 resulting source files matched its manifest byte-for-byte.

## Baseline archives

Original tarball: linux-5.6.19-pyradef_5.6.19-pyradef.orig.tar.gz

Original SHA-256: 4ca8561817b9bae8b462b5fa67a6b38a9f77d50c0661bc5f00b8d795f208ba6d

Debian source diff: linux-5.6.19-pyradef_5.6.19-pyradef-10.diff.gz

Diff SHA-256: 28533e0045f53eda551ea48bd6c22570905f5b26ea0c4cef432a224d308a8270

## Source file SHA-256 fingerprints

Each pair records the SHA-256 of the baseline file and the modified file. “New file” denotes the newly added board DTS. Full 64-character digests are included for reproducibility.

01  arch/arm/Makefile

BASE  7f364f93fee2324ad9a2c92bcfcd26424b68c43e0d0ea8dd2aa888a206a93800

NEW   b0011d8bf734900ef258cfe9d1a8ba12d13e66a9cbf2cf5638b1b1709659bedd

02  arch/arm/boot/dts/Makefile

BASE  208b0ff26e17fabbbf5add7af51d20744ed5c5566528d792d2d8f0636540c956

NEW   293b0ee9fa34b9ec69ba52cabe3f90dacefb0194d91824c2a545c048d9e35ec5

03  arch/arm/boot/dts/omap5-letux-cortex15-v5.3+pyra-v5.3-m4test.dts

BASE  (new file)

NEW   48855d76ae2428bad78bad97ca8eae8f50fd815ab20e8b0c161f70ff6a064da1

04  arch/arm/boot/dts/omap5.dtsi

BASE  0cc541af315cbff678b0ed7821ca1807def7009e03f39f56a3c37a6ceff725e9

NEW   57192bac9b668f22205fc1c7296528e127ffa4e43c0d349675dd4d60a5d2dbc5

05  arch/arm/configs/pyra_defconfig

BASE  ad7e6ebc382d1def0de4f32f4850b696061af8b302b244a9bb4c3e091151d829

NEW   d7ca694bdc3584957c062b24230936d7b64e66b6d2fbf3c4f3d1650c5eede6bb

06  drivers/gpu/drm/omapdrm/omap_dmm_tiler.c

BASE  8c16917a90eb8dd61d03a40e0fb383086a25ca6fc41d45bc86a5cd30f6c2b87d

NEW   4482e2b0fb8334547c4a2b2f09c4507d9857cb4140b83a28ac776228cbde4b6e

07  drivers/iommu/omap-iommu.c

BASE  1eac9415a73844bb686e0461ca0015e290be399f8ed5bc2b7769ae40d3e9dbbb

NEW   12adf92a7718d6945f9aa6c1e9701afa0e2e434bd4c7fb5f5bd19f764a513495

08  drivers/mmc/host/omap_hsmmc.c

BASE  a65a0d553d14a446aa8c3647c6e3754873c2dbaa7f9779a88831e1d985f63b1c

NEW   20ae9ee02af9dfaab27f1b8c2b084e3baa3e9ab5a68e293c4dd164b533adb7a7

09  drivers/power/supply/bq2429x_charger.c

BASE  5a64958e773fe39bb0233d81f52106067087b07b0d9eb93e516037683199ffdd

NEW   02ccedb014c411ab6742b1fd8469601e7a9b51c5832d545044ac723492dcd2c9

10  drivers/remoteproc/Kconfig

BASE  f9a97c433d85aecdb0227ec219fe1b7cceee818bf7644c92633beaf2f8c4f4ec

NEW   a4ae88077ba0ddfd250dccfb0eb95d1d9a5606fdedaca1b0e388efdcc2a1888f

11  drivers/remoteproc/omap_remoteproc.c

BASE  0bc8dc2bf5334553767d3e1360813f8d164e32b5e4302defd4890fbcb9aa8869

NEW   28bf710d654cd99e048cdbd2a89078b20c44804b952fa3f106cba6c34d3baa06

12  drivers/remoteproc/omap_remoteproc.h

BASE  36e71c0cf1941819c160dfa03aeb4dda4666110eb194cad298166de3adf7df26

NEW   5b592ec8acf6e0d42b426606b4b4262f596970c2e86d10038d54900db6265400

13  drivers/soc/ti/omap_prm.c

BASE  cceb0f3b5a02feb8ee70f9e74c625660e11b59e0ef2267b8eb380b23a463d813

NEW   e49a610d32c1089d8e7531f19573dcb6249276d864d1d567b028d83465646a5a

14  drivers/usb/dwc3/dwc3-omap.c

BASE  464b4239af35d9ccb06576e0fa99bfd6744c481a66280fa87abbfad011ed2f12

NEW   1bc0e7309da430285d62db7a6835122f27a8e21f23f0401abff78fb35ae0e7b6

15  security/selinux/include/classmap.h

BASE  8eb03ae562a9dfb0ad0b4927e1af50b1177909e20fecc082128ecce3be22c6a6

NEW   7d454deadeb350b92ac490656fce5f95d86a1d9080ee6a61ba473c60a1d72c4b

## Complete line-by-line source patch

For an exact line-by-line representation of every source change, see the companion text artifact “Pyra_Kernel_5.6.19_pyradef_M4_v6_Complete_Changes_EN.txt”, Appendix A. This report prioritizes a navigable engineering overview while retaining the complete source-file inventory and integrity fingerprints.